EMBEDDED SYSTEMS — PROJECT REVIEW 2

Autonomous Water Surface Cleaning Robot

with Active Water Filtration

Platform: ESP32-CAM  |  RTOS: FreeRTOS  |  MCU: Dual-core Xtensa LX6 @ 240MHz

# 1. System Overview & Architecture

This project implements an Autonomous Water Surface Cleaning Robot for ponds and lakes. The robot navigates the water surface using rear-mounted propellers, collects floating garbage passively via a front-mounted mesh scoop, and simultaneously filters the top water layer using a pump-based filtration system. Water quality is monitored in real time using a TDS sensor and a turbidity sensor — these readings determine when and where filtering is needed. The system is controlled wirelessly via a WiFi web interface hosted on the ESP32-CAM.

The novelty of this project lies in the combination of passive physical garbage collection with active surface water filtration. Most existing robots only do one or the other. By monitoring TDS and turbidity, the robot can also quantify the effectiveness of its filtration — before/after readings prove the filtration is working.

## 1.1 System Block Diagram

┌──────────────────────────────────────────────────────────────────┐

│                    POWER SUPPLY  (7.4V LiPo)                    │

│    ┌─────────────┐                   ┌────────────────┐         │

│    │  3.3V reg   │                   │    5V reg      │         │

└────┴──────┬──────┴───────────────────┴──────┬─────────┴─────────┘

│                                  │

┌──────▼──────────────────────────────────────────────┐

│                  ESP32-CAM (MCU)                     │

│   Core 1: Motor/Sensor control tasks                 │

│   Core 0: WiFi + Camera stream tasks                 │

│   Peripherals: GPIO, PWM(LEDC), ADC, UART, I2C      │

└────┬──────┬──────┬──────┬──────┬───────┬────────────┘

│      │      │      │      │       │

┌────▼─┐ ┌──▼──┐ ┌─▼──┐ ┌▼────┐ ┌▼────┐ ┌▼──────┐

│L298N │ │L298N│ │ TDS│ │TURB │ │WiFi │ │Camera │

│  #1  │ │  #2 │ │SEN │ │SEN  │ │AP   │ │OV2640 │

│PROPS │ │PUMP │ │ADC │ │ADC  │ │     │ │MJPEG  │

└──┬───┘ └──┬──┘ └────┘ └─────┘ └─────┘ └───────┘

┌────▼──┐ ┌───▼──┐

│2x DC  │ │DC    │

│Motor  │ │Pump  │

│+Props │ │Motor │

└───────┘ └──────┘

[PROPULSION]  [FILTER]

## 1.2 Component Summary

| Component | Qty | Role | Interface |
| --- | --- | --- | --- |
| ESP32-CAM (AI-Thinker) | 1 | Central MCU + WiFi + Camera | All GPIO / ADC / PWM |
| L298N Dual Motor Driver | 2 | Drive props + pump motor | GPIO + PWM |
| DC Motor + Propeller | 2 | Boat propulsion | L298N output |
| DC Water Pump Motor | 1 | Surface water filtration | L298N output |
| TDS Sensor Module | 1 | Water quality — dissolved solids | Analog → ADC pin |
| Turbidity Sensor Module | 1 | Water clarity — suspended particles | Analog → ADC pin |
| Mesh / Net scoop (front) | 1 | Passive garbage collection | Mechanical only |
| OV2640 Camera (onboard) | 1 | Live video monitoring | DVP + SCCB (onboard) |
| LiPo 7.4V battery pack | 1 | Power supply | Direct to rails |

# 2. Circuit Diagram

All grounds must share a common GND reference. The L298N onboard 5V regulator (available when Vs > 7V) can power the ESP32 via its 3.3V onboard regulator. Add a 100µF electrolytic capacitor across motor supply rails to suppress switching noise.

BATTERY 7.4V (+) ──────┬──────────────────────────────────┐

│                                  │

┌────▼───────────────┐  ┌──────────────▼──────────────┐

│   L298N #1 (PROPS) │  │   L298N #2 (PUMP)           │

GPIO 12 ──────────►│ IN1                │  │ IN1 ◄─────────── GPIO 16    │

GPIO 13 ──────────►│ IN2  Motor A+/A-  ├─►│ IN2 ◄─────────── GPIO 17    │

GPIO 14 (PWM) ────►│ ENA               │  │ ENA ── tie to 5V (full spd) │

GPIO 15 ──────────►│ IN3                │  │ Motor Out+ ──► Pump Motor + │

GPIO  2 ──────────►│ IN4  Motor B+/B-  ├─►│ Motor Out- ──► Pump Motor - │

GPIO  1 (PWM) ────►│ ENB               │  └─────────────────────────────┘

│ Out A+ ──► Left Motor +

│ Out A- ──► Left Motor -

│ Out B+ ──► Right Motor +

│ Out B- ──► Right Motor -

└────────────────────┘

TDS Sensor:

VCC ──► 3.3V                Turbidity Sensor:

GND ──► GND                   VCC ──► 5V

A-OUT──► GPIO 34 (ADC)         GND ──► GND

A-OUT──► GPIO 35 (ADC)

ESP32-CAM:

5V  ◄── L298N #1 onboard 5V reg (or USB for testing only)

GND ──► Common GND

NOTE: GPIO 34, 35 are input-only ADC pins — ideal for sensors.

NOTE: Add 100µF cap between Vs and GND on each L298N.

# 3. Component Analysis

## 3.1 MCU — ESP32-CAM

### Working Principle

The ESP32-CAM integrates a dual-core Xtensa LX6 processor (up to 240 MHz), 520KB SRAM, 4MB PSRAM, 4MB Flash, WiFi 802.11 b/g/n, and an OV2640 camera on a single module. Core 0 is dedicated to the WiFi/BT protocol stack. Core 1 is free for application tasks — motor control, sensor reading, FSM logic. GPIO, LEDC (PWM), ADC, UART, I2C, and SPI peripherals are available on the exposed pins.

### On-Chip Peripherals Used

- LEDC (PWM): 4 channels — 2 for prop motor speed, 1 for pump speed control

- GPIO Digital Out: 6 pins for L298N direction signals (IN1–IN4 x2 drivers)

- ADC: 2 pins (GPIO 34, 35) for TDS and turbidity analog sensor signals

- UART0: Debug serial output via TX/RX for sensor value monitoring

- WiFi: AP mode — robot hosts its own hotspot, phone browser connects and sends commands

### Pin Assignment

| GPIO | Connected To | Type | Function |
| --- | --- | --- | --- |
| 12 | L298N-1 IN1 | Digital Out | Left prop — direction A |
| 13 | L298N-1 IN2 | Digital Out | Left prop — direction B |
| 14 | L298N-1 ENA | PWM (LEDC) | Left prop — speed control |
| 15 | L298N-1 IN3 | Digital Out | Right prop — direction A |
| 2 | L298N-1 IN4 | Digital Out | Right prop — direction B |
| 1 | L298N-1 ENB | PWM (LEDC) | Right prop — speed control |
| 16 | L298N-2 IN1 | Digital Out | Pump motor — enable |
| 17 | L298N-2 IN2 | Digital Out | Pump motor — direction |
| 34 | TDS sensor A-OUT | ADC Input | Water quality — dissolved solids |
| 35 | Turbidity A-OUT | ADC Input | Water clarity — turbidity |

### Libraries

- arduino-esp32 core — GPIO, LEDC PWM, ADC, UART, WiFi

- ESPAsyncWebServer + AsyncTCP — non-blocking HTTP server for control UI and sensor data

- esp32-camera — OV2640 camera init and MJPEG stream

- ArduinoJSON — parse/build JSON for sensor readings sent to browser

### Interrupt vs Polling Strategy

Motor control and sensor reading are handled in periodic FreeRTOS tasks (polling every 20ms and 500ms respectively). WiFi HTTP request handling uses event-driven callbacks — the ESP32 WiFi stack fires a callback on each incoming HTTP request, which queues a command for the motor control task. No hardware interrupts are needed for the basic system.

## 3.2 L298N Dual Motor Driver

### Working Principle

The L298N is a dual H-bridge motor driver. Each H-bridge takes two direction input signals (IN1/IN2) and an enable signal (ENA with PWM) to drive one DC motor in either direction at variable speed. It handles up to 2A per channel, making it suitable for small DC motors and pump motors. The onboard 5V regulator outputs logic power when motor supply Vs >= 7V.

### Communication Protocol

No serial protocol — pure GPIO logic level control. Direction is set by writing HIGH/LOW to IN1 and IN2. Speed is controlled by the PWM duty cycle on ENA (0 = stopped, 255 = full speed). Calibration involves measuring actual motor RPM at different duty cycles and adjusting software offsets so both propeller motors produce equal thrust.

### Pseudocode — Motor Control

// Move forward: both motors same speed

void moveForward(int speed) {

digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW);  // Left fwd

ledcWrite(CH_LEFT, speed);

digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW);  // Right fwd

ledcWrite(CH_RIGHT, speed);

}

// Turn: reduce one motor speed

void turnLeft(int speed) {

ledcWrite(CH_LEFT,  speed / 2);  // Slow left

ledcWrite(CH_RIGHT, speed);       // Full right

}

// Stop: brake mode

void stopMotors() {

digitalWrite(IN1, HIGH); digitalWrite(IN2, HIGH);

ledcWrite(CH_LEFT, 0); ledcWrite(CH_RIGHT, 0);

}

### Levels of Coding — Motor Speed (ENA/PWM)

| Level | Code |
| --- | --- |
| API | ledcWrite(channel, dutyCycle); // 0-255 |
| SFR | LEDC.channel[0].duty.duty = (dutyCycle << 4); LEDC.channel[0].conf0.sig_out_en = 1; |
| Direct Address | *(volatile uint32_t*)(LEDC_BASE + 0x14) = (dutyCycle << 4); |
| ASM (inline) | __asm__ volatile("movi a2, 0x3FF59014; s32i %0, a2, 0" : : "r"(dutyCycle << 4)); |

### Optimization — Time, Space, Energy

- Time: Use IRAM_ATTR on the motor update function to run from IRAM (faster than Flash) — avoids cache miss overhead in time-critical PWM updates.

- Space: One generic setMotor(id, dir, speed) function replaces duplicated code — saves code Flash space.

- Energy: When stationary, set PWM to 0 and put motors in brake mode (not float) — eliminates back-EMF oscillation and reduces idle current draw.

## 3.3 TDS Sensor Module

### Working Principle

TDS (Total Dissolved Solids) sensors measure the electrical conductivity of water by passing a small AC signal between two probe electrodes. More dissolved substances (salts, organic waste, fertilizers) increase conductivity. The sensor outputs an analog voltage proportional to TDS in ppm (parts per million). Clean water typically reads < 300 ppm; polluted pond water may read 500–2000 ppm. This is the primary metric for demonstrating that the filtration system is actually working — readings drop after water passes through the filter mesh.

### Interfacing

3 pins: VCC (3.3V), GND, and one analog output pin connected to ESP32 GPIO 34 (ADC1 Channel 6). The ESP32 ADC has 12-bit resolution (0–4095 counts mapped to 0–3.3V). The sensor board has a temperature compensation circuit onboard. No calibration protocol is needed beyond a one-point offset calibration using known-TDS water (e.g., tap water at ~200 ppm).

### Reading Logic

float readTDS() {

int raw = analogRead(TDS_PIN);           // 0-4095

float voltage = raw * (3.3 / 4095.0);    // convert to volts

float tds_ppm = (133.42 * voltage * voltage * voltage

- 255.86 * voltage * voltage

+ 857.39 * voltage) * 0.5;

return tds_ppm;

}

// Sample every 500ms (soft real-time, no strict deadline)

### Interrupt vs Polling

Polling — sampled every 500ms in the SensorTask. ADC reads are fast (~100µs) and don't benefit from interrupt-driven access. Averaging 10 consecutive readings reduces ADC noise common on ESP32.

## 3.4 Turbidity Sensor Module

### Working Principle

A turbidity sensor shines an infrared LED through water and measures how much light reaches the detector. Suspended particles (silt, algae, organic matter) scatter and absorb the light. Higher turbidity = fewer particles reaching the detector = lower analog output voltage. Output is analog voltage (or optionally digital threshold). Turbidity is measured in NTU (Nephelometric Turbidity Units). Tap water: < 1 NTU. Polluted pond water: 5–100+ NTU. This complements TDS — TDS catches dissolved pollutants, turbidity catches suspended ones.

### Interfacing

3 pins: VCC (5V), GND, analog output to GPIO 35 (ADC1 Channel 7). Note this sensor needs 5V supply (not 3.3V) but the output can be read by the ESP32 ADC as it outputs 0–4.5V scaled down. Use a voltage divider (10kΩ + 4.7kΩ) to scale to 0–3.3V before connecting to ADC pin.

float readTurbidity() {

int raw = analogRead(TURB_PIN);

float voltage = raw * (3.3 / 4095.0) * (14.7 / 4.7); // undo divider

// Approximate NTU from voltage (sensor-specific curve)

float ntu = -1120.4 * voltage * voltage + 5742.3 * voltage - 4352.9;

return (ntu < 0) ? 0 : ntu;

}

## 3.5 DC Water Pump Motor + Filtration

### Working Principle

A small submersible or inline DC pump (6–12V) draws water from just below the surface and pushes it through a mesh/filter material. Filtered water exits back to the pond; trapped particles stay in the filter. The pump is driven by L298N #2 with IN1=HIGH, IN2=LOW (fixed direction) and ENA tied HIGH (or PWM-controlled for variable flow rate). The pump runs whenever the robot is active and TDS/turbidity readings exceed threshold.

### Pseudocode — Adaptive Filtration

void filteringTask() {

float tds = readTDS();

float ntu = readTurbidity();

if (tds > TDS_THRESHOLD || ntu > NTU_THRESHOLD) {

startPump(200);  // Run pump at 80% speed

} else {

stopPump();      // Water is clean enough

}

}

// TDS_THRESHOLD = 400 ppm, NTU_THRESHOLD = 10

## 3.6 OV2640 Camera (Onboard) + WiFi Control

### Working Principle

The OV2640 communicates via SCCB (I2C-like) for register configuration and DVP parallel interface for pixel data — both are handled internally by the ESP32-CAM module. The camera captures JPEG frames stored in PSRAM. These are streamed as MJPEG over HTTP to any browser on the WiFi network. Resolution is set to QVGA (320×240) for low latency.

### WiFi Control Interface

The ESP32 runs in Access Point (AP) mode — it creates its own WiFi hotspot named 'WaterBot'. A phone or laptop connects to this network and opens a browser to 192.168.4.1. The web page shows the live camera feed, sensor readings (TDS and NTU), and buttons to control movement and pump. Commands are sent as HTTP GET requests: /cmd?action=forward, /cmd?action=stop, etc. Sensor data is returned as JSON: {"tds": 420, "ntu": 12}.

- Library: ESPAsyncWebServer + esp32-camera

- Protocol: HTTP/1.1 over TCP/IP over WiFi 802.11n

- Stream endpoint: GET /stream → MJPEG multipart stream

- Command endpoint: GET /cmd?action=X → JSON response

- Sensor endpoint: GET /sensors → JSON with TDS and turbidity values

# 4. System Modeling

## 4.1 Finite State Machine

The robot's behaviour is modeled as a 5-state reactive FSM. Transitions are triggered either by WiFi commands from the operator or by sensor threshold crossings.

┌────────────┐

┌─────────────► BOOT       │

│             │ Init HW,   │

│             │ Start WiFi │

│             └─────┬──────┘

│           init_ok │

│                   ▼

fault ───┤            ┌──────────────┐

│  ◄─stop────│    IDLE      │────filter_cmd──►┐

│            │  all motors  │                  │

│            │  stop, read  │◄──stop──┐        │

│            │  sensors     │         │        ▼

│            └──────┬───────┘    ┌────┴───────────────┐

│          fwd/turn │             │    FILTERING       │

│                   ▼             │  pump ON, sensors  │

│            ┌──────────────┐     │  read every 500ms  │

│            │   MOVING     │     └────────────────────┘

│            │  props ON,   │

│            │  mesh scoops │

│            └──────────────┘

│

▼

┌──────────┐

│  FAULT   │──reset──► BOOT

│ all stop │

└──────────┘

### State Table

| State | Entry Action | Trigger to Exit | Exit Action |
| --- | --- | --- | --- |
| BOOT | Init GPIO, PWM, ADC, WiFi AP, Camera | init_ok | Start tasks, enter IDLE |
| IDLE | Stop all motors; read sensors every 500ms | WiFi cmd | Execute command |
| MOVING | Set prop PWM per direction command | STOP cmd | Props off, go IDLE |
| FILTERING | Start pump motor; log TDS/turbidity | FILTER_STOP cmd | Pump off, go IDLE |
| FAULT | Emergency stop all outputs | RESET cmd | Re-init, go BOOT |

## 4.2 Sequential Programming Model (SESE)

The main loop follows a Single-Entry Single-Exit structure. All state transitions are handled within one pass of the loop. ISR-based approach is not used for the main FSM — FreeRTOS task + queue is sufficient and cleaner.

// ENTRY

initHardware();     // GPIO, PWM channels, ADC, WiFi, Camera

createTasks();      // FreeRTOS tasks for motor, sensor, camera

state = BOOT;

LOOP:               // SESE body — never exits in embedded

cmd = peekQueue(); // non-blocking

if (cmd != NULL):

nextState = transition(state, cmd);

if (nextState != state):

runExitAction(state);

runEntryAction(nextState);

state = nextState;

runStateAction(state);   // periodic work

vTaskDelay(20ms);

GOTO LOOP

// EXIT — unreachable

## 4.3 Concurrent Programming Model — FreeRTOS Tasks

Four FreeRTOS tasks run concurrently, pinned to specific cores to avoid WiFi stack interference with real-time tasks.

| Task | Core | Priority | Period | Responsibility |
| --- | --- | --- | --- | --- |
| MotorControlTask | 1 | 4 (high) | 20ms | Read cmdQueue → set GPIO/PWM for motors |
| SensorTask | 1 | 3 | 500ms | Read TDS + turbidity ADC → update shared vars |
| WiFiCommandTask | 0 | 3 | async | Receive HTTP cmd → push to cmdQueue |
| CameraStreamTask | 0 | 2 (low) | 100ms | Capture JPEG frame → serve MJPEG stream |

Inter-Task Communication: WiFiCommandTask sends commands to MotorControlTask via an xQueue (depth 10, sizeof Command_t). SensorTask writes TDS/turbidity to global volatile variables protected by a mutex. CameraStreamTask accesses the camera frame buffer via a semaphore to prevent concurrent access.

## 4.4 Object-Oriented Class Model

┌─────────────────────────────────┐

│            WaterBot              │

│─────────────────────────────────│

│ - state: RobotState              │

│ - leftMotor: MotorDriver         │

│ - rightMotor: MotorDriver        │

│ - pump: MotorDriver              │

│ - tds: TDSSensor                 │

│ - turbidity: TurbiditySensor     │

│ - camera: CameraModule           │

│ - wifi: WiFiController           │

│─────────────────────────────────│

│ + init()                         │

│ + handleCommand(cmd: Command)    │

│ + getSensorData(): SensorData    │

└───────────────┬─────────────────┘

│ has-a

┌────────────┼──────────────────────┐

▼            ▼                       ▼

┌──────────┐ ┌────────────┐ ┌────────────────────┐

│MotorDrvr │ │ TDSSensor  │ │ WiFiController     │

│──────────│ │────────────│ │────────────────────│

│-in1,in2  │ │-pin:int    │ │-ssid, password     │

│-enaCh:int│ │-lastPPM    │ │-server: AsyncServer│

│──────────│ │────────────│ │────────────────────│

│+forward()│ │+readPPM()  │ │+startAP()          │

│+reverse()│ │+isAboveThr │ │+onCommand(cb)      │

│+stop()   │ └────────────┘ │+sendSensorData()   │

│+setSpeed │                └────────────────────┘

└──────────┘

## 4.5 DES Model — Race Conditions & Deadlock Analysis

| Scenario | Hazard | Mitigation |
| --- | --- | --- |
| WiFiTask and MotorTask both access cmdQueue simultaneously | Race condition | xQueue is FreeRTOS thread-safe — built-in mutual exclusion |
| SensorTask writes TDS while WiFiTask reads it for HTTP response | Race condition on shared variable | xSemaphoreTake/Give mutex wrapping sensor read/write |
| CameraTask holds frame buffer while WiFiTask tries to send it | Deadlock risk | Timeout on semaphore (1000ms); release frame buffer and retry |
| Two LEDC channels assigned to same timer accidentally | Resource conflict | Assign each PWM channel to a different LEDC timer at init; verified in code |

# 5. Data Flow Graph (DFG) Analysis

DFG analysis for the motor speed computation — calculating left and right propeller PWM from a speed and turn input.

## 5.1 HLL Statements (Single Assignment Form)

// Inputs: speed (0-255), turn (-100 to +100)

int t1 = turn * 128 / 100;        // scale turn to PWM units

int t2 = speed + t1;              // right motor value

int t3 = speed - t1;              // left motor value

int rightPWM = clamp(t2, 0, 255); // bound to valid range

int leftPWM  = clamp(t3, 0, 255); // bound to valid range

| DFG Property | Analysis |
| --- | --- |
| Variables | 7: speed, turn, t1, t2, t3, rightPWM, leftPWM |
| LHS reuse | None — single assignment form; no variable appears on LHS twice |
| Operations | 1× multiply, 1× divide, 1× add, 1× subtract, 2× clamp = 6 ops |
| Acyclic | Yes — no feedback; graph is a DAG (directed acyclic graph) |
| Independent ops | t2 and t3 are independent (both depend only on t1 and speed) |
| Levels | Level 1: t1 ¦ Level 2: t2, t3 (parallel) ¦ Level 3: clamp both (parallel) |
| Reordering benefit | t2 and t3 can execute in parallel on out-of-order hardware |

## 5.2 DFG Diagram

speed ──────────────────┐      turn

│        │

│    [× 128/100]

│        │ t1

│    ┌───┴────────────┐

│    │                 │

[+]◄┘◄───┘             [−]◄──── speed

t2                     t3

│                       │

[clamp 0-255]          [clamp 0-255]

│                       │

rightPWM               leftPWM

Level 1 (serial):   t1 = turn × 128 / 100

Level 2 (parallel): t2 = speed + t1   ||   t3 = speed - t1

Level 3 (parallel): rightPWM = clamp(t2) || leftPWM = clamp(t3)

## 5.3 Hardware Efficiency

- Register efficiency: Only 3 temporary registers needed (t1, t2, t3). With -O2, compiler keeps all in registers without spilling to stack.

- Cache efficiency: All variables are local — fit entirely in L1 cache (16KB on ESP32). Zero cache misses.

- Pipeline efficiency: No data hazard exists between Level 2 and Level 3 — clamp reads the output of add/sub, which is already committed. No pipeline stall.

## 5.4 CDFG — Flowchart & Worst-Case Execution Path

[START: command received from queue]

│

▼

<cmd.type?>

│         │            │           │

MOVE      STOP       FILTER       RESET

│         │            │           │

[compute   [stop all   [start/stop  [reinit

PWM L,R]   motors]    pump based    HW]

│         │          on sensors]   │

[set GPIO  [set GPIO      │          [reboot

+ LEDC]    = 0]       [setGPIO      tasks]

│         │          pump]         │

END       END          │           END

END

Worst-case path: RESET path → ~15ms (WiFi teardown + reinit)

Normal path (MOVE): ~0.5ms (queue read + 4× GPIO + 2× ledcWrite)

Filter path: ~1ms (queue read + ADC read x2 + GPIO pump ctrl)

# 6. Code Optimizations

## 6.1 Compiler Optimization Flags

In the ESP32 Arduino environment, optimization level is set in platform.txt or via build flags. Recommended for this project: -O2 (full optimization without size increase). The -O2 flag enables instruction scheduling, loop unrolling, inlining, and register allocation — exactly what benefits the motor PWM update loop.

| Flag | Effect | Applicability |
| --- | --- | --- |
| -O0 | No optimization — largest binary, slowest | Debug only |
| -O2 | Full optimization — recommended | Production build |
| -Os | Optimize for size — reduces Flash usage | If Flash is tight |
| IRAM_ATTR | Place function in SRAM — avoids Flash cache miss | Motor update, ISRs |

## 6.2 Static Analysis

The ESP32 compiler (GCC Xtensa) provides static analysis warnings with -Wall -Wextra flags. Key issues to detect and fix before runtime: uninitialized GPIO pins floating HIGH (can trigger motors on boot), integer overflow in PWM arithmetic (speed+turn > 255), missing volatile keyword on variables shared between FreeRTOS tasks (compiler may optimize away reads that are actually changed by another task), and stack overflow in tasks (use uxTaskGetStackHighWaterMark() to check).

## 6.3 Profiling — Runtime Behaviour

Use the ESP32 cycle counter to measure actual execution times:

// Profile any function

uint32_t t0 = xthal_get_ccount();

myFunction();

uint32_t elapsed_cycles = xthal_get_ccount() - t0;

float ms = elapsed_cycles / 240000.0f;  // at 240MHz

Serial.printf("Execution: %.3f ms\n", ms);

Expected findings: Motor control update < 0.5ms (not a bottleneck). ADC sensor read ~0.1ms each. Camera frame capture ~30ms (biggest single cost). WiFi HTTP response ~5–80ms depending on payload size. The camera stream task is the dominant CPU load — keep it on Core 0 to not interfere with motor control on Core 1.

# 7. Real-Time Requirements Analysis

## 7.1 Task Categorization

| Task | RT Category | Relative Urgency | Why |
| --- | --- | --- | --- |
| Motor PWM update | Hard Real-Time | Highest | Missing 20ms deadline causes physical jerk or motor overcurrent |
| Pump on/off control | Soft Real-Time | Medium | 100ms late start has no catastrophic effect on filtration |
| WiFi command receive | Soft Real-Time | Medium | Operator perceives >200ms lag as unresponsive but no safety risk |
| Sensor data read (TDS/Turbidity) | Soft Real-Time | Low-Medium | 500ms period — slow-changing physical measurement |
| Camera MJPEG stream | Soft Real-Time | Low | Frame drops acceptable — monitoring only, not safety-critical |

## 7.2 Workload Characterization

### Synchronous (Periodic) Events

| Event | Period | Exec Time | Deadline |
| --- | --- | --- | --- |
| Motor PWM update | 20 ms | ~0.5 ms | 20 ms (hard) |
| Sensor read (TDS + Turbidity) | 500 ms | ~1 ms | 500 ms (soft) |
| Camera frame capture | 100 ms | ~30 ms | 100 ms (soft) |

### Asynchronous Events

| Event | Arrival Pattern | Required Response |
| --- | --- | --- |
| WiFi HTTP command (move/stop/filter) | Sporadic, operator-triggered | Queue within 10ms; execute within 50ms |
| TDS crosses threshold (auto-pump) | Sporadic, environment-triggered | Start pump within 1 sensor period (500ms) |
| WiFi disconnect | Rare | Stop all motors (safety); attempt reconnect |

## 7.3 Timing Parameters & Phases of Periodic Events

Periodic tasks are started with a phase offset to prevent all tasks waking simultaneously at t=0 (which would cause a CPU spike every 100ms LCM boundary):

t = 0ms:   MotorControlTask starts (period 20ms)

t = 3ms:   CameraStreamTask starts (period 100ms) — offset by 3ms

t = 7ms:   SensorTask starts (period 500ms) — offset by 7ms

LCM(20, 100, 500) = 500ms hyperperiod

In each 500ms window:

Motor task runs 25 times   (25 × 0.5ms = 12.5ms)

Camera task runs 5 times   (5 × 30ms  = 150ms)

Sensor task runs 1 time    (1 × 1ms   = 1ms)

Total CPU: 163.5ms / 500ms = 32.7% utilization (feasible)

## 7.4 Scheduler Analysis

Rate Monotonic Scheduling (RMS) is applied. In RMS, tasks with shorter periods get higher priority. Feasibility is verified by CPU utilization bound: U = Σ(Ci/Ti) ≤ n(2^(1/n) − 1).

Task             Ci      Ti      Ci/Ti

Motor control:  0.5ms   20ms    0.025

Camera stream:  30ms   100ms    0.300

Sensor read:     1ms   500ms    0.002

─────────────────────────────────────

Total U = 0.327

For n=3 tasks: bound = 3(2^(1/3) - 1) = 0.780

U = 0.327 < 0.780  →  Schedule is VALID and FEASIBLE ✓

| Scheduler | Suitable? | Reasoning |
| --- | --- | --- |
| Clock-driven / Rate Monotonic | YES (primary) | Periodic tasks dominate; RMS proven feasible above |
| Event-driven (for WiFi cmds) | YES (secondary) | Aperiodic WiFi events handled via FreeRTOS queue within clock-driven framework |
| Single processor (ESP32 Core 1) | YES | RT tasks on Core 1; WiFi/camera on Core 0 — effectively dual processor |
| Multi-processor (external MCU) | Not needed | Dual-core ESP32 suffices for this workload |

## 7.5 FreeRTOS Thread Implementation

### Thread Creation

xTaskCreatePinnedToCore(

motorControlTask,  // function

"MotorCtrl",       // name

2048,              // stack bytes

NULL,              // params

4,                 // priority

&motorHandle,      // handle

1                  // core 1

);

xTaskCreatePinnedToCore(sensorTask, "Sensor", 2048, NULL, 3, NULL, 1);

xTaskCreatePinnedToCore(wifiTask,   "WiFi",   4096, NULL, 3, NULL, 0);

xTaskCreatePinnedToCore(cameraTask, "Camera", 8192, NULL, 2, NULL, 0);

### Thread Scheduling

FreeRTOS uses preemptive priority scheduling. Motor task (priority 4) will preempt sensor (priority 3), which preempts camera (priority 2). The scheduler runs every tick (1ms by default). vTaskDelayUntil() is used instead of vTaskDelay() for period-accurate wakeup — this prevents drift over time.

### Thread Synchronization

// Command queue — WiFi task to Motor task

QueueHandle_t cmdQueue = xQueueCreate(10, sizeof(Command_t));

xQueueSend(cmdQueue, &cmd, 0);           // producer (WiFi task)

xQueueReceive(cmdQueue, &cmd, 0);        // consumer (motor task)

// Mutex — protect shared sensor variables

SemaphoreHandle_t sensorMutex = xSemaphoreCreateMutex();

xSemaphoreTake(sensorMutex, portMAX_DELAY);

sensorData.tds = readTDS();            // critical section

xSemaphoreGive(sensorMutex);

### Thread Cancellation

// Soft cancellation via task notification

xTaskNotify(motorHandle, SHUTDOWN_BIT, eSetBits);

// Inside motorControlTask — check each cycle:

uint32_t bits;

if (xTaskNotifyWait(0, SHUTDOWN_BIT, &bits, 0) == pdTRUE)

if (bits & SHUTDOWN_BIT) { stopAllMotors(); vTaskDelete(NULL); }

# 8. Summary

This report presents the complete embedded systems design for an Autonomous Water Surface Cleaning Robot. The ESP32-CAM serves as the sole MCU, leveraging its dual-core architecture for concurrent real-time motor control and WiFi communication. The robot passively collects surface garbage via a front-mounted mesh scoop pushed forward by rear propellers, while simultaneously filtering the top water layer using a pump-based system.

The key technical novelty is the integration of TDS and turbidity sensors to monitor water quality in real time. These sensors quantify exactly how polluted the water is before and after filtering, providing measurable proof of filtration effectiveness. The pump activates automatically when either reading exceeds a configurable threshold — making the filtration intelligent and energy-efficient rather than running continuously.

The system is validated through: a Rate Monotonic feasibility analysis (U = 0.327 < 0.780 bound), a 5-state FSM covering all operational scenarios including fault handling, a 4-task FreeRTOS concurrent model with safe inter-task communication via queues and mutexes, DFG analysis showing the motor computation is acyclic, parallelizable, and register-efficient, and a DES hazard analysis identifying and mitigating all potential race conditions and deadlocks.